MAT255 / MAT257

Exercise 1

This exercise is a tutorial to test your installation of ECLIPSE and let you get acquainted with ECLIPSE postprocessing software.

Copy the data file Eks1.DATA to your preferred folder (don’t use the eclipse installation folder). Sugggestion: Make a “parent folder” mat255, and put the data file in a subfolder Ex1 or something similar.

A)

Open the ECLIPSE launcher panel. Start ECLIPSE by upper-leftmost button labeled ECLIPSE. You get to choose startup directory. Choose the one where you put Eks1.DATA (Ex1-folder). Select the DATA file, and press RUN.

ECLIPSE should then start running your file in a command window. Check that the run completed without problems. (You should get an error summary at the end of the run – check that this consists of 1 comments, 0 for the rest).

Close the command window and check the Ex1-folder. Confirm that the following files are present:

EKS1.PRT

EKS1.INIT		EKS1.EGRID

EKS1.SMSPEC	EKS1.RSSPEC

EKS1.X0000		EKS1.X0001		EKS1.X0002		EKS1.X0003

EKS1.S0001		EKS1.S0002		EKS1.S0003

B)

Open the “Office” Button (at middle of launcher panel) (Select appropriate folder)

The ECLIPSE Office window should appear after a while.

Press the button “Result” at the left hand side of the window. A new window appears.

All further action is in this window.

Select (Menu) File  Open  Summary  Load All Vectors

(File dialog window  select Eks1.SMSPEC, which should be the only one available)

The upper left window now contains a list of loaded data vectors, sorted by first letter in name. Select the ‘+’ by the F, to get a list of loaded Field data. Double-click the box FOPR (Field Oil Production Rate). A figure should appear in the right window, containing the selected graph (appears empty, but there is a green line at the top!).

Next, right-click the box by FPR (Field Pressure), and select “Add to current graph”.

Average pressure should then appear in the same graph as the oil rate curve.

There’s a lot more to do in Office, and you are encouraged to play around with it for a while, but this actually suffices for our main use, which is to look at results.

Close Office.

C)

Open FloViz button (next to Office in launcher panel). After a while a FloViz window with a dummy grid should appear. Select

(Menu) File  Open  Eclipse (File dialog – select EKS1.GRID  New window with loading options; just choose OK).

Our reservoir model should appear in the main window. The two rightmost icons below the menu (+z and –z) are used to change the magnification in the z-direction. Click “+z” a couple of times and notice the change.

Check that the mouse pointer is hand-shaped. Hold left mouse button while moving the mouse  rotates the model in the 3D-window. Pressing the SHIFT key in addition will move the model.

Select (Menu) Scene  Grid  IJK Slice (dialog window).

Select I: 1 – 8 (first click “1”, then hold SHIFT key while clicking “8”).

Select J: 2

Go to flag “K-slicing”: Select 1 and 3 (First “1”, then Ctrl-key while selecting “3”).

Resulting picture gives an indication that we may select which parts of the model to view in a quite flexible manner.

Select (Menu) Scene  Grid  Property (New window: Property window).

In Property Window select “Initial” radio-button (at top), in list below, select DEPTH – note picture, then select PERMX, then PORO.

With PORO selected:

In the upper left corner of the main window are two symbols, “hand” and “arrow”. Should be “hand” now. Select “arrow”. Notice that the mouse pointer changes to an arrow. Click on cell (1, 1, 1) on the model. Notice the info-field at lower left of the main window, which now should read:

Eks1: global 1 1 1 (1) PORO = 0.30000.

I.e., the value of the chosen property for the selected cell is shown. Often very useful to check that our input is correct.

Now select cell (1, 3, 3) in the same manner. Confirm that porosity is 0.18 in this cell.

Go back to the Property Window, and select the “Recurrent” radio button (at top). This opens a list of the dynamic variables. Select PRESSURE.

In main window, upper right part are some buttons resembling VCR-controls,

(<<   |<      >   >|    >>). These are used to select time steps: From left:

first – previous – STOP – PLAY – next – last.

Press the << button (first time step). The time info window (lower right of main window) should read 1. Jan 2004 (which was our start date). Press “next” (>|) and note how the pressure colour changes. Note also the time info window, which now should be 1. Jul. 2004. Repeat to see 1. Jan 2005 and 1 Jul 2005 (last time step).

Click “play” (>) – animation a bit on the fast side?

Select (Menu) View  Timesteps (Timestep window). In Timestep Window click “Options”, set “Animation Delay” to 0.5 seconds, and click “Apply”. Click “play” button – should be far better suited for study. Lastly, click the “recirculation” symbol in the Timestep Window, followed by the play button.

You should now have seen some of the most important features in FloViz, but there’s a lot more for the curious. Play around with the program to your own pleasure, and close when content.

Exercise 2

Take the Eks1.DATA  example as a starting point. (Advice: Copy it to a new subfolder, e.g. Ex2)

The goal of this exercise is to get acquainted with the most common eclipse keywords.

A)

All input data is in field units.

Convert the data to metric, i.e. feet -> m, lb -> kg, stb -> m3, psi -> bars. Consult the manual for finding out what the data items mean, and what units they are in.

Unit conversion formulas can be found in “Eclipse Technical Description”, chapter on Units (74 in 2012-version).  (Use approximate conversions, rounding off to a nice number)

Change the grid to 5 blocks in the y-direction, DY = 100m. The two topmost layers should be 10m thick, the lowest layer 20m.

Porosity: Three different regions, high, low, and zero.

 = 0 in the box i = 4-7, j = 1-5, k = 3.

Low porosity for i = 8-10, k = 2-3, j = 2-4.

The rest is high porosity.

In the high-porosity region, use  = 0.3, PERMX = 800, NTG = 1

In the low-porosity region, use  = 0.2, PERMX = 70, NTG = 0.7

PERMY = PERMX, PERMZ = 0.1*PERMX.

The well should still be centered in the j-direction, i.e. change to j = 3.

Run this case, correct for errors, look at the results in Office (or other program if you prefer).

B)

Change start date to 1.1.2005. Change DATES, with milestones every second month the first year, every third month the second year, and every sixth month for another two years.

Set a producing oil rate such that approximately 10% of initial oil in place is produced each year (find the initial oil volume in the .PRT file). Note how long the simulation runs before oil rate is as good as zero, and end simulation at about that time.

Run the simulation. Use FloViz to look at how pressure and saturation changes with time.